Base Layers

├── Roads

├── Buildings

Civil Layers

├── Trench

├── Duct & Sub-Duct

├── Chamber & Handhole

├── Pole

Cable Layers

├── Feeder Cable

├── Distribution Cable

├── Garden Cable

Equipment Layers

├── MFG-Node

├── PDP

├── Splitter

├── FAT/FDP

Demand Layers

├── Premises

├── Homes Passed

├── Utilization

1. Base Layers

These provide reference information.

- Administrative boundaries

- Roads and streets

- Building footprints

- Parcels / plots

- Utility corridors

- Existing telecom infrastructure

- Satellite / aerial imagery

- Topography (if relevant)

2. Civil Infrastructure Layers

Trench Layer Attributes

| Attribute | Example |
| --- | --- |
| Trench ID | TR-001 |
| Type | Open Cut / Micro Trench / HDD |
| Usage type | Feeder/Distribution/Garden |
| Length | 250 m |
| Width | 300 mm |
| Depth | 900 mm |
| Surface Type | Asphalt / Concrete / Footpath |
| Reinstatement Type | Road / Sidewalk |
| Status | Existing / Planned / Proposed |

Duct Layer:

| Attribute | Example |
| --- | --- |
| Duct ID | DCT-001 |
| Parent Trench ID | TR-001 |
| Duct Type | 4-Way HDPE |
| Diameter | 110 mm |
| Number of Ways | 4 |
| Length | 250 m |
| Start Chamber | MH-001 |
| End Chamber | MH-002 |
| Occupancy | 60% |
| Spare Capacity | 40% |
| Status | Existing / Planned / Proposed |

Simple Rule for HLD

| Asset | Network Segment | Primary Use |
| --- | --- | --- |
| Manhole | Feeder | Large cables, large duct banks, backbone access |
| Chamber | Feeder + Distribution | Splicing, branching, cable pulling |
| Handhole | Distribution + Garden | FAT/FDP access, Garden cable connections |

| Attribute | Example |
| --- | --- |
| Structure ID | MH-001 |
| Type | Manhole / Chamber / Handhole |
| Parent Trench ID | TR-001 |
| Connected Duct Count | 4 |
| Size | 1500×1200 mm |
| Associated Equipment | PREMISES/OBJECTS-001 / FAT-001 |
| Used Capacity | 60% |
| Spare Capacity | 40% |
| Status | Existing / Planned / Proposed |

Pole Layer (for aerial deployment)

| Attribute | Example |
| --- | --- |
| Pole ID | PL-001 |
| Pole Type | Utility / Telecom |
| Material | Concrete |
| Height | 9 m / 7m |
| Connected Cable Count | 3 |
| Associated Equipment | FAT-001 |
| Used Capacity | 60% |
| Spare Capacity | 40% |
| Status | Existing / Planned / Proposed |

3. Fiber Cable Layers

| Attribute | Example |
| --- | --- |
| Cable ID | FC-001 |
| Cable Name | Chennai-Feeder-01 |
| Cable Type | Feeder / Distribution / Garden |
| Fiber Count | 288F |
| Length | 5 km |
| Installation Type | Underground |
| Source Node | MFG-01 |
| Route ID | RT-001 |
| Service Area | Sholinganallur |
| Status | Existing / Planned / Proposed |
| Utilization (%) | 65% |
| Owner | Network Operations |

Feeder Cable Layer

MFG → PDP

Attributes:

- Cable ID

- Fiber count (96F,144F, 288F, etc.)

- Route length

- Utilization

Distribution Cable Layer

PDP → Premises/Objects

Attributes:

- Cable size

- Fiber count(48F,24F,12F etc.)

- Route ID

Garden Cable Layer

DP/FAT → Customer

Attributes:

- Customer ID

- Garden type

- Length

5. Distribution Equipment Layers

| Attribute | Example |
| --- | --- |
| Equipment ID | EQ-001 |
| Equipment Name | MFG-01 |
| Equipment Type | MFG / PREMISES/OBJECTS / Splitter / FAT / ONT |
| Layer | Core / Distribution / Access / Customer |
| Location | Central Office / Street Cabinet / Building |
| Latitude | 12.8991 |
| Longitude | 80.2279 |
| Capacity | 16 PON Ports / 288 Fibers |
| Split Ratio | 1:8 / 1:16 / 1:32 / 1:64 |
| Connected Fibers | 288 |
| Status | Existing / Planned / Proposed |
| Vendor | Nokia / Huawei / ZTE / Calix |
| Power Requirement | Yes / No |
| Maintenance Zone | Zone-A |

| Equipment Type | Typical Location | Input | Output | Capacity |
| --- | --- | --- | --- | --- |
| MFG | Central Office | Core Network | Feeder Fiber | 8-16-32 PON Ports |
| PREMISES/OBJECTS | Outdoor Cabinet | Feeder Cable | Distribution Cable | Up to 1,152 Fibers |
| FAT | Pole / Pedestal / Building | Distribution Cable | Garden Cable | 8-48 Customer Ports |
| Optical Splitter | PREMISES/OBJECTS/FAT | 1 Fiber | 8/16/32/64 Fibers | Based on Split Ratio |
| ONT/ONU | Customer Premises | Garden Fiber | Ethernet/Wi-Fi | 1 Subscriber |

6. Customer & Demand Layers

Premises Layer

Potential serviceable buildings.

Attributes:

- Building ID

- Unit count

- Customer type

Demand Layer

Forecast demand and take rate.

Attributes:

- Homes Passed

- Homes Connected

- Business customers

Network Hierarchy Layer

Example:

Plain Text

MFG - Node

└── Feeder Cable – 288f, 144f,

└── PDP-

└── Splitter

└── Distribution Cable – 48f, 24f

└── FAT/FDP

Splitter

└── Garden Cable

└── ONT - Customer

Show more lines